DTC Monitor & Analyzer (Simulation-based)

Design a diagnostic tool that monitors and analyzes Diagnostic Trouble Codes (DTCs) using a simulation-based environment aligned with OBD-II/UDS standards. The system focuses on two main capabilities:

DTC Monitoring:

• Simulate and track diagnostic trouble codes in real-time, including setting, updating, and clearing DTCs.

• Support OBD-II / UDS standard fault code formats (P, C, B, U code categories).

• Live DTC feed with status indicators (active, pending, stored) and severity levels.

DTC Analysis:

• Analyze DTC data to provide insights such as occurrence frequency, historical trends, and fault categorization.

• Identify recurring faults and correlate co-occurring DTCs across simulated sessions.

• Fault categorization by system (engine, transmission, body, chassis, network).

Output Requests:

• Real-time DTC monitor dashboard — Live feed of active/pending/stored DTCs with code, description, severity, and timestamp; supports manual set/clear actions for simulation control.

• DTC analysis report — Occurrence frequency chart per fault code, historical trend over simulated time range, fault categorization breakdown by system; filterable by category and time period.

• Top recurring faults view — Ranked list of most frequently triggered DTCs with occurrence count, first/last seen timestamps, and co-occurrence correlation with other active codes.

• Session export — Full DTC log for a simulated session exportable as JSON or CSV; includes all events (set, update, clear) with timestamps and metadata for offline analysis.

• Extensible simulator interface — Documented abstraction layer separating DTC simulation engine from analysis logic; interface contract defined so a real OBD-II / ECU source can be swapped in without changing the analysis layer.