Voice-Controlled Assistant

Develop a voice-controlled assistant that enables drivers to interact with vehicle functions and services through natural voice commands, enhancing safety and convenience in automotive environments.

Key functional groups include:

• Car Control & General Assistant Functions: Support voice commands for controlling in-vehicle features such as door lock/unlock, volume adjustment, media control, and climate settings. The assistant also provides general services including weather information, hands-free calling (free talk), and basic infotainment interactions.

• Delivery Vehicle Voice Scenarios: Provide specialized voice interaction flows tailored for delivery use cases, such as route navigation, package status updates, stop-by-stop instructions, and hands-free task execution to support drivers during delivery operations.

• Performance & Latency Optimization: Optimize voice processing for fast response time and low latency, ensuring smooth real-time interaction. The system supports efficient intent recognition and command execution, with potential hybrid architecture (edge + cloud) to balance performance and accuracy.

Output Requests:

• Voice interaction demo — car control scenarios — Live demo covering at least 5 voice commands across door control, volume, media, and climate; HMI reflects state change immediately upon command execution.

• Delivery voice flow demo — End-to-end walkthrough of a simulated delivery route: driver activates assistant, receives stop-by-stop instructions, queries package status, and confirms task completion — all hands-free.

• Latency benchmark report — Measured end-to-end response time (wake word → intent recognized → action executed) across at least 20 test utterances; includes comparison between edge-only and hybrid (edge + cloud) configurations if applicable.

• System architecture diagram — Documented intent recognition pipeline from voice input → ASR → NLU → command dispatcher → vehicle system integration; shows extension points for adding new intents or vehicle signals.

• Working APK on AAOS emulator — Installable on Android Automotive OS AVD; demonstrates both car control and delivery scenarios with real voice input and visible HMI response.